Alternate Activity 5 –Building From a 2D Image

Now that you have looked at one way to create a detailed house model from a plan in activity 5, there are alternate methods to accomplish the same thing.

Using the floor plan image you have chosen, you can import it into Sketchup and resize it using a similar technique to that shown in the videos below.

Remember to save your SketchUp model frequently. Save it as “U:\BBT\Activity 5”.

- Videos

Importing 2D Images

http://www.youtube.com/watch?v=MtqJh-DUG30&feature=plcp&context=C44e900fVDvjVQa1PpcFNk2m2DBHemvivj7dneEijxXe8XBHPWRk4%3D

Creating Walls

http://www.youtube.com/watch?v=FsnT8KN8xIM&feature=relmfu

Doors and Windows

https://www.youtube.com/watch?v=W4CMKV062e8

There are several dimensions which are conventionally used on houses built in Canada. Keep in mind the following sizes:

Exterior walls = 6”

Interior walls = 4”

Closet depth = 2’

Hallways minimum 4’ wide

Stairs minimum 3’ wide

Exterior doors = 3’ minimum

Interior doors = commonly 2’6” – 2’ 8” wide

Stair treads = commonly 10” deep

Stair riser = commonly 7” tall

Remember to save your SketchUp model frequently. Save it as “U:\BBT\Activity 5”.